Assignment-1

NAME:Muskan kumari(2501010265)

AIM:Implement NOT, AND,OR gates using NAND gates using NAND2TETRIS simulation SCHEMATIC Diagram, Boolean Expression and Truth Table

Code: CHIP Not {

IN in;

OUT out;

PARTS:

Nand(a=in, b=in, out=out);

}

CODE:

CHIP And {

IN a, b;

OUT out;

PARTS:

Nand(a=a, b=b, out=temp);

Nand(a=temp, b=temp, out=out);

}

CODE: CHIP Or {

IN a, b;

OUT out;

PARTS:

Nand(a=a, b=a, out=nota);

Nand(a=b, b=b, out=notb);

Nand(a=nota, b=notb, out=out);

}